# Filter Driers — Product Catalog & SEO Content

Catalog Path: Filter Driers

Supplier: Ningbo HVACR Net Refrigeration Equipment Co., Ltd. (HVACR NET)

Website: www.hvacrnet.com

Contact: Echo YANG · Echo@hvacrnet.com

MOQ: 1 piece (small-batch wholesale supported)

## Product 1: BFK Series — Bidirectional Solid Core Filter Drier (SAE Flare)

Meta Title: Bidirectional Filter Drier, SAE Flare (BFK) | Refrigeration

(59 chars)

Meta Description: Bidirectional solid core filter drier with SAE flare connections, 3/8"–3/4", 80% molecular sieve + 20% alumina, MWP 45 bar. 10 models. MOQ 1 pc.

(140 chars)

### Product Name

Bidirectional Solid Core Filter Drier — SAE Flare (BFK Series)

### Short Description

A bidirectional filter drier featuring a solid core of 80% 3A molecular sieve and 20% activated alumina for superior drying and deacidification, the BFK Series offers SAE flare threaded connections across 10 models covering port sizes from 3/8" to 3/4". It is designed for HFC and HCFC refrigeration systems using POE or PAG oils.

### Product Description

• Bidirectional dry filter designed for installation on the liquid piping of refrigeration and air conditioning systems to dry refrigerant (absorb moisture from the system) and filter solid contaminants, ensuring efficient and safe system operation.

• Filter core composed of 80% 3A molecular sieve and 20% activated alumina with 25μm filter screen.

• Suitable for HFC and HCFC refrigerants and synthetic polyester oil (POE or PAG) systems.

• Bidirectional flow design allows installation in either direction on the liquid line, providing flexible piping options.

### Key Features

• 80% 3A molecular sieve + 20% activated alumina solid core delivers powerful drying capacity and deacidification capability.

• 25μm filter fineness maintains high-efficiency filtration with minimal pressure drop.

• Corrosion-resistant powder coating finish suitable for various environments.

• Standard SAE flare threaded connection; custom designs available upon request.

• Can be mounted in any desirable direction for flexible installation.

• Bidirectional flow design eliminates installation direction constraints.

### Available Models

| Model | Port Size (Inch) | Thread M | Overall Size L×D (mm) |
| --- | --- | --- | --- |
| BFK-083 | 3/8 | 5/8-18UNF | 158 × 66.5 |
| BFK-084 | 1/2 | 3/4-16UNF | 165 × 66.5 |
| BFK-085 | 5/8 | 7/8-14UNF | 176 × 66.5 |
| BFK-163 | 3/8 | 5/8-18UNF | 170 × 79 |
| BFK-164 | 1/2 | 3/4-16UNF | 179 × 79 |
| BFK-165 | 5/8 | 7/8-14UNF | 186 × 79 |
| BFK-303 | 3/8 | 5/8-18UNF | 245 × 79 |
| BFK-304 | 1/2 | 3/4-16UNF | 254 × 79 |
| BFK-305 | 5/8 | 7/8-14UNF | 261 × 79 |
| BFK-306 | 3/4 | 1-1/16-14UNS | 265 × 79 |

### Technical Parameters

| Parameter | Value |
| --- | --- |
| Series Code | BFK |
| Filter Core Type | Solid Core |
| Filter Media | 80% 3A Molecular Sieve + 20% Activated Alumina |
| Filter Fineness | 25μm |
| Flow Direction | Bidirectional |
| Refrigerant Compatibility | HCFC, HFC |
| Medium Temperature Range | −40 °C ~ +120 °C |
| MWP (Maximum Working Pressure) | 4.5 MPa (45 bar) |
| Maximum Test Pressure | 6.75 MPa |
| Connection Type | SAE Flare |
| Surface Coating | Corrosion-resistant powder coating |
| Mounting Direction | Any direction |

## Product 2: BFK Series — Bidirectional Solid Core Filter Drier (Brazed ODF)

Meta Title: Bidirectional Filter Drier, Brazed ODF (BFK) | Refrigeration

(60 chars)

Meta Description: Bidirectional solid core filter drier with brazed ODF connections, 3/8"–1-1/8", 80% molecular sieve + 20% alumina, MWP 45 bar. 12 models. MOQ 1 pc.

(142 chars)

### Product Name

Bidirectional Solid Core Filter Drier — Brazed ODF (BFK Series)

### Short Description

Offering the same 80% molecular sieve + 20% activated alumina solid core for powerful drying and acid removal, the BFK Series in brazed ODF connection provides a larger size range than the SAE flare variant. With 12 models covering port sizes from 3/8" to 1-1/8", it serves HFC and HCFC refrigeration systems.

### Product Description

• Bidirectional dry filter designed for installation on the liquid piping of refrigeration and air conditioning systems to dry refrigerant (absorb moisture from the system) and filter solid contaminants, ensuring efficient and safe system operation.

• Filter core composed of 80% 3A molecular sieve and 20% activated alumina with 25μm filter screen.

• Suitable for HFC and HCFC refrigerants and synthetic polyester oil (POE or PAG) systems.

• Bidirectional flow design allows installation in either direction on the liquid line, providing flexible piping options.

### Key Features

• 80% 3A molecular sieve + 20% activated alumina solid core delivers powerful drying capacity and deacidification capability.

• 25μm filter fineness maintains high-efficiency filtration with minimal pressure drop.

• Corrosion-resistant powder coating finish suitable for various environments.

• Brazed ODF (Outside Diameter Female) connection for permanent, leak-free installations.

• Can be mounted in any desirable direction for flexible installation.

• Extended port size range up to 1-1/8" for larger capacity systems.

### Available Models

| Model | Port Size (Inch) | Overall Size L×D (mm) |
| --- | --- | --- |
| BFK-083S | 3/8 | 141 × 66.5 |
| BFK-084S | 1/2 | 149 × 66.5 |
| BFK-085S | 5/8 | 149 × 66.5 |
| BFK-163S | 3/8 | 153 × 79 |
| BFK-164S | 1/2 | 161 × 79 |
| BFK-165S | 5/8 | 161 × 79 |
| BFK-303S | 3/8 | 228 × 79 |
| BFK-304S | 1/2 | 236 × 79 |
| BFK-305S | 5/8 | 236 × 79 |
| BFK-306S | 3/4 | 241 × 79 |
| BFK-307S | 7/8 | 247 × 79 |
| BFK-309S | 1-1/8 | 256 × 79 |

### Technical Parameters

| Parameter | Value |
| --- | --- |
| Series Code | BFK |
| Filter Core Type | Solid Core |
| Filter Media | 80% 3A Molecular Sieve + 20% Activated Alumina |
| Filter Fineness | 25μm |
| Flow Direction | Bidirectional |
| Refrigerant Compatibility | HCFC, HFC |
| Medium Temperature Range | −40 °C ~ +120 °C |
| MWP (Maximum Working Pressure) | 4.5 MPa (45 bar) |
| Maximum Test Pressure | 6.75 MPa |
| Connection Type | Brazed (ODF) |
| Surface Coating | Corrosion-resistant powder coating |
| Mounting Direction | Any direction |

## Product 3: DFS Series — Unidirectional Solid Core Filter Drier (SAE Flare)

Meta Title: Unidirectional Filter Drier, SAE Flare (DFS) | Refrigeration

(60 chars)

Meta Description: Unidirectional solid core filter drier with SAE flare connections, 1/4"–3/4", 100% 3A molecular sieve, MWP 45 bar. 13 models. MOQ 1 pc.

(135 chars)

### Product Name

Unidirectional Solid Core Filter Drier — SAE Flare (DFS Series)

### Short Description

A unidirectional filter drier featuring a 100% 3A molecular sieve solid core for powerful moisture absorption without powder migration, the DFS Series offers SAE flare threaded connections across 13 models covering port sizes from 1/4" to 3/4". It is ideal for HFC and HCFC refrigeration systems requiring precise flow direction control.

### Product Description

• Unidirectional dry filter designed for installation on the liquid piping of refrigeration and air conditioning systems to dry refrigerant (absorb moisture from the system) and filter solid contaminants, ensuring efficient and safe system operation.

• Filter core composed of 100% 3A molecular sieve with 25μm filter screen.

• Suitable for HFC and HCFC refrigerants and synthetic polyester oil (POE or PAG) systems.

• Unidirectional flow design with integrated flow direction indicator ensures correct installation.

### Key Features

• 100% 3A molecular sieve solid core delivers powerful drying capacity with no powder migration, ensuring long-term stable performance.

• 25μm filter fineness maintains high-efficiency filtration with minimal pressure drop.

• Corrosion-resistant powder coating finish suitable for various environments.

• Standard SAE flare threaded connection; custom designs available upon request.

• Can be mounted in any desirable direction for flexible installation.

• Flow direction arrow clearly marked on body for foolproof installation.

### Available Models

| Model | Port Size (Inch) | Thread M | Overall Size L×D (mm) |
| --- | --- | --- | --- |
| DFS-032 | 1/4 | 7/16-20UNF | 110 × 43 |
| DFS-033 | 3/8 | 5/8-18UNF | 119 × 43 |
| DFS-052 | 1/4 | 7/16-20UNF | 118 × 54 |
| DFS-053 | 3/8 | 5/8-18UNF | 127 × 54 |
| DFS-083 | 3/8 | 5/8-18UNF | 152 × 54 |
| DFS-084 | 1/2 | 3/4-16UNF | 159 × 54 |
| DFS-085 | 5/8 | 7/8-14UNF | 170 × 54 |
| DFS-163 | 3/8 | 5/8-18UNF | 159 × 75 |
| DFS-164 | 1/2 | 3/4-16UNF | 166 × 75 |
| DFS-165 | 5/8 | 7/8-14UNF | 177 × 75 |
| DFS-304 | 1/2 | 3/4-16UNF | 256 × 79 |
| DFS-305 | 5/8 | 7/8-14UNF | 267 × 79 |
| DFS-306 | 3/4 | 1-1/16-14UNS | 268 × 79 |

### Technical Parameters

| Parameter | Value |
| --- | --- |
| Series Code | DFS |
| Filter Core Type | Solid Core |
| Filter Media | 100% 3A Molecular Sieve |
| Filter Fineness | 25μm |
| Flow Direction | Unidirectional |
| Refrigerant Compatibility | HCFC, HFC |
| Medium Temperature Range | −40 °C ~ +120 °C |
| MWP (Maximum Working Pressure) | 4.5 MPa (45 bar) |
| Maximum Test Pressure | 6.75 MPa |
| Connection Type | SAE Flare |
| Surface Coating | Corrosion-resistant powder coating |
| Mounting Direction | Any direction |

## Product 4: DFS Series — Unidirectional Solid Core Filter Drier (Brazed ODF)

Meta Title: Unidirectional Filter Drier, Brazed ODF | Refrigeration

(55 chars)

Meta Description: Unidirectional solid core filter drier with brazed ODF connections, 1/4"–1-1/8", 100% 3A molecular sieve, MWP 45 bar. 15 models. MOQ 1 pc.

(137 chars)

### Product Name

Unidirectional Solid Core Filter Drier — Brazed ODF (DFS Series)

### Short Description

Providing the largest size range in the DFS solid core lineup, this unidirectional filter drier in brazed ODF connection offers a 100% 3A molecular sieve solid core for pure moisture absorption. With 15 models covering port sizes from 1/4" to 1-1/8", it serves HFC and HCFC refrigeration systems.

### Product Description

• Unidirectional dry filter designed for installation on the liquid piping of refrigeration and air conditioning systems to dry refrigerant (absorb moisture from the system) and filter solid contaminants, ensuring efficient and safe system operation.

• Filter core composed of 100% 3A molecular sieve with 25μm filter screen.

• Suitable for HFC and HCFC refrigerants and synthetic polyester oil (POE or PAG) systems.

• Unidirectional flow design with integrated flow direction indicator ensures correct installation.

### Key Features

• 100% 3A molecular sieve solid core delivers powerful drying capacity with no powder migration, ensuring long-term stable performance.

• 25μm filter fineness maintains high-efficiency filtration with minimal pressure drop.

• Corrosion-resistant powder coating finish suitable for various environments.

• Brazed ODF (Outside Diameter Female) connection for permanent, leak-free installations.

• Can be mounted in any desirable direction for flexible installation.

• Extended port size range up to 1-1/8" for larger capacity systems.

### Available Models

| Model | Port Size (Inch) | Overall Size L×D (mm) |
| --- | --- | --- |
| DFS-032S | 1/4 | 99 × 43 |
| DFS-033S | 3/8 | 101 × 43 |
| DFS-052S | 1/4 | 107 × 54 |
| DFS-053S | 3/8 | 109 × 54 |
| DFS-083S | 3/8 | 134 × 54 |
| DFS-084S | 1/2 | 142 × 54 |
| DFS-085S | 5/8 | 142 × 54 |
| DFS-163S | 3/8 | 142 × 75 |
| DFS-164S | 1/2 | 150 × 75 |
| DFS-165S | 5/8 | 150 × 75 |
| DFS-304S | 1/2 | 239 × 79 |
| DFS-305S | 5/8 | 239 × 79 |
| DFS-306S | 3/4 | 244 × 79 |
| DFS-307S | 7/8 | 250 × 79 |
| DFS-309S | 1-1/8 | 259 × 79 |

### Technical Parameters

| Parameter | Value |
| --- | --- |
| Series Code | DFS |
| Filter Core Type | Solid Core |
| Filter Media | 100% 3A Molecular Sieve |
| Filter Fineness | 25μm |
| Flow Direction | Unidirectional |
| Refrigerant Compatibility | HCFC, HFC |
| Medium Temperature Range | −40 °C ~ +120 °C |
| MWP (Maximum Working Pressure) | 4.5 MPa (45 bar) |
| Maximum Test Pressure | 6.75 MPa |
| Connection Type | Brazed (ODF) |
| Surface Coating | Corrosion-resistant powder coating |
| Mounting Direction | Any direction |

## Product 5: DFS Series — Replaceable Core Filter Drier

Meta Title: Replaceable Core Filter Drier, Brazed ODF | Refrigeration

(57 chars)

Meta Description: Replaceable core filter drier with brazed ODF connections, 5/8"–2-5/8", 100% 3A molecular sieve, MWP 42 bar. 19 models, 1–3 cores. MOQ 1 pc.

(141 chars)

### Product Name

Replaceable Core Filter Drier — Brazed ODF (DFS Series)

### Short Description

Designed for system cleanup after compressor burnout, this replaceable core filter drier offers cost-effective maintenance with a replaceable filter element. Available in 1-core, 2-core, and 3-core configurations with 19 models covering port sizes from 5/8" to 2-5/8", the DFS Series serves large commercial and industrial refrigeration systems using HCFC, HFC, or CFC refrigerants.

### Product Description

• Replaceable core filter drier designed for installation on the liquid and vapor piping of refrigeration and air conditioning systems to effectively remove residual contaminants, protecting the compressor and extending its service life.

• The filter element is replaceable, enabling cost-effective maintenance without replacing the entire housing.

• Specifically engineered to absorb large amounts of moisture entering the system in the event of compressor motor burnout, making it the preferred choice for system cleanup and recovery.

• Compatible with HCFC, HFC, and CFC refrigerants.

### Key Features

• Removable structure for convenient installation and maintenance — replace only the filter core, not the entire unit.

• 100 mesh outlet filter screen provides secondary filtration protection.

• Full pass-through design minimizes pressure drop for optimal system efficiency.

• Steel casing with copper pipe welded connections; high-frequency welding with flanging structure ensures superior welding reliability.

• Modified Teflon sealing material guarantees perfect sealing performance.

• Epoxy resin electrostatic coating provides long-lasting corrosion protection.

• Available in 1-core (420 m²), 2-core (840 m²), and 3-core (1260 m²) configurations for systems of different capacities.

### Available Models

| Model | Port Size (Inch) | Cores | Filtering Surface (m²) | Overall Size L×D (mm) |
| --- | --- | --- | --- | --- |
| DFS-485S | 5/8 | 1 | 420 | 234 × 114 |
| DFS-486S | 3/4 | 1 | 420 | 234 × 114 |
| DFS-487S | 7/8 | 1 | 420 | 234 × 114 |
| DFS-489S | 1-1/8 | 1 | 420 | 236 × 114 |
| DFS-4811S | 1-3/8 | 1 | 420 | 240 × 114 |
| DFS-4813S | 1-5/8 | 1 | 420 | 242 × 114 |
| DFS-4817S | 2-1/8 | 1 | 420 | 246 × 114 |
| DFS-4821S | 2-5/8 | 1 | 420 | 244 × 114 |
| DFS-967S | 7/8 | 2 | 840 | 376 × 114 |
| DFS-969S | 1-1/8 | 2 | 840 | 378 × 114 |
| DFS-9611S | 1-3/8 | 2 | 840 | 382 × 114 |
| DFS-9613S | 1-5/8 | 2 | 840 | 384 × 114 |
| DFS-9617S | 2-1/8 | 2 | 840 | 388 × 114 |
| DFS-9621S | 2-5/8 | 2 | 840 | 386 × 114 |
| DFS-1449S | 1-1/8 | 3 | 1260 | 520 × 114 |
| DFS-14411S | 1-3/8 | 3 | 1260 | 524 × 114 |
| DFS-14413S | 1-5/8 | 3 | 1260 | 526 × 114 |
| DFS-14417S | 2-1/8 | 3 | 1260 | 530 × 114 |
| DFS-14421S | 2-5/8 | 3 | 1260 | 528 × 114 |

### Technical Parameters

| Parameter | Value |
| --- | --- |
| Series Code | DFS (Replaceable) |
| Filter Core Type | Replaceable Core |
| Filter Media | 100% 3A Molecular Sieve + 25μm Filter Pad |
| Filter Fineness | 25μm (filter pad) / 100 mesh (outlet screen) |
| Flow Direction | Unidirectional |
| Refrigerant Compatibility | HCFC, HFC, CFC |
| Medium Temperature Range | −35 °C ~ +70 °C |
| MWP (Maximum Working Pressure) | 4.2 MPa (42 bar) |
| Maximum Test Pressure | 6.3 MPa |
| Connection Type | Brazed (ODF) |
| Surface Coating | Epoxy resin electrostatic coating |
| Mounting Direction | Any direction |
| Number of Cores | 1, 2, or 3 |
| Filtering Surface Area | 420 m² (1 core) / 840 m² (2 cores) / 1260 m² (3 cores) |

## 5-Product Comparison Table

| # | Product Name | Series | Core Type | Filter Media | Flow | Connection | Port Size | MWP | Temp Range | Models |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | BFK Bidirectional (SAE Flare) | BFK | Solid Core | 80% Mol. Sieve + 20% Alumina | Bidirectional | SAE Flare | 3/8"~3/4" | 45 bar | −40~+120 °C | 10 |
| 2 | BFK Bidirectional (ODF) | BFK | Solid Core | 80% Mol. Sieve + 20% Alumina | Bidirectional | Brazed ODF | 3/8"~1-1/8" | 45 bar | −40~+120 °C | 12 |
| 3 | DFS Unidirectional (SAE Flare) | DFS | Solid Core | 100% 3A Mol. Sieve | Unidirectional | SAE Flare | 1/4"~3/4" | 45 bar | −40~+120 °C | 13 |
| 4 | DFS Unidirectional (ODF) | DFS | Solid Core | 100% 3A Mol. Sieve | Unidirectional | Brazed ODF | 1/4"~1-1/8" | 45 bar | −40~+120 °C | 15 |
| 5 | DFS Replaceable Core | DFS-R | Replaceable | 100% 3A Mol. Sieve | Unidirectional | Brazed ODF | 5/8"~2-5/8" | 42 bar | −35~+70 °C | 19 |

## SEO Keywords

### Product 1: BFK Bidirectional Solid Core (SAE Flare)

• bidirectional filter drier

• SAE flare filter drier

• bidirectional dry filter

• solid core filter drier

• molecular sieve filter drier

• activated alumina filter drier

• liquid line filter drier

• refrigeration filter drier SAE

• BFK filter drier

• HVACR filter drier

### Product 2: BFK Bidirectional Solid Core (ODF)

• bidirectional filter drier brazed

• ODF filter drier

• bidirectional dry filter brazed

• solid core filter drier ODF

• welded filter drier

• molecular sieve filter drier ODF

• liquid line filter drier brazed

• refrigeration filter drier ODF

• BFK filter drier ODF

• HVACR filter drier brazed

### Product 3: DFS Unidirectional Solid Core (SAE Flare)

• unidirectional filter drier

• SAE flare filter drier

• unidirectional dry filter

• solid core filter drier

• 100% molecular sieve filter drier

• liquid line filter drier

• refrigeration filter drier SAE

• DFS filter drier

• HVACR filter drier

• unidirectional filter drier SAE

### Product 4: DFS Unidirectional Solid Core (ODF)

• unidirectional filter drier brazed

• ODF filter drier

• unidirectional dry filter brazed

• solid core filter drier ODF

• welded filter drier

• 100% molecular sieve filter drier ODF

• liquid line filter drier brazed

• refrigeration filter drier ODF

• DFS filter drier ODF

• HVACR filter drier brazed

### Product 5: DFS Replaceable Core

• replaceable core filter drier

• replaceable filter drier

• filter drier replacement core

• compressor burnout filter drier

• system cleanup filter drier

• large capacity filter drier

• industrial filter drier

• multi-core filter drier

• DFS replaceable filter drier

• HVACR replaceable filter drier

## GEO Optimization Notes

This content is structured to maximize discoverability and accurate citation by AI-powered search engines (Google AI Overviews, Perplexity, ChatGPT Search, Bing Copilot, etc.) through the following strategies:

### 1. Structured Data Tables

Each product includes a Technical Parameters table with consistent parameter fields (Filter Core Type, Filter Media, Flow Direction, MWP, Connection Type, etc.). AI engines extract and compare structured tabular data with high fidelity. The 5-Product Comparison Table at the end provides a single reference point for cross-product comparison queries (e.g., "What is the difference between bidirectional and unidirectional filter driers?" or "Which filter drier has the highest filtering surface area?").

### 2. Specific Parameter Values

All technical specifications use precise numerical values (e.g., "MWP 4.5 MPa (45 bar)", "−40 °C to +120 °C", "25μm filter fineness", "420 m² filtering surface") rather than vague descriptors. AI engines prioritize content with concrete data points when answering factual queries such as "What is the maximum working pressure of BFK filter driers?"

### 3. Standard Full-Name References

Industry standards and technical terms are written in full on first use: "Maximum Working Pressure (MWP)", "SAE Flare", "Brazed (ODF — Outside Diameter Female)", "3A Molecular Sieve", "Activated Alumina", "Polyester Oil (POE)", "Polyalkylene Glycol (PAG)". This enables AI engines to match both abbreviated and full-form search queries.

### 4. Manufacturer-Sourced Product Descriptions

Each product includes a Product Description and Key Features section sourced from the manufacturer's professional technical documentation. These sections describe specific application scenarios (liquid piping, vapor piping), flow direction characteristics (bidirectional vs. unidirectional), filter media composition, and engineering features (corrosion-resistant coating, mounting flexibility, replaceable core design), enabling AI search engines to accurately understand and cite product application contexts when answering queries such as "Which filter drier is used for compressor burnout cleanup?" or "What is the difference between solid core and replaceable core filter driers?"

### 5. Entity-Rich Context

Company name, brand, contact information, and product series codes (BFK, DFS, DFS Replaceable) are consistently referenced, enabling AI engines to build entity relationships between the brand "HVACR NET", the product category "filter drier", and specific model series.

### 6. Cross-Product Comparisons

Product descriptions include explicit cross-references (e.g., "BFK vs. DFS", "SAE Flare vs. ODF", "Solid Core vs. Replaceable Core") that anticipate comparison-based AI queries. The comparison table provides a structured answer source for queries like "Which HVACR NET filter drier supports the largest port size?" or "What is the difference between BFK and DFS filter driers?"

## About HVACR NET

Ningbo HVACR Net Refrigeration Equipment Co., Ltd. is a B2B wholesale supplier of refrigeration and HVACR components, based in Ningbo, China. With a 1,000 m² warehouse holding extensive stock, we support small-batch orders starting from 1 piece (MOQ = 1) with flexible delivery timelines.

• Website: www.hvacrnet.com

• Contact: Echo YANG

• Email: Echo@hvacrnet.com

• Product Range: Filter driers, solenoid valves, expansion valves, ball valves, sight glasses, copper tubes, insulation tubes, and refrigeration accessories

This document contains 5 filter drier product category pages with SEO-optimized meta tags, structured specifications, and GEO-friendly content formatting. All technical parameters are sourced from HVACR NET product data. © HVACR NET.

本内容由 Coze AI 生成，请遵循相关法律法规及《人工智能生成合成内容标识办法》使用与传播。